311         ALIGATOR ALUMINIUM GUTTERS

	Manufacturer: Marley Alutec, Unit 1 (G-H), Hudson Road,
	Elms Farm Industrial Estate, Bedford MK41 0LZ
	Tel: 01234 359438, Fax: 01234 357199.
	Email: enquiries@marleyalutec.co.uk
	Reference: Aligator snap fit Boxer aluminium rainwater system
	Profile: Box
	Size: 135x100mm 
Flow Performance: As stated in Alutec literature.

Outlet Size: 63mmØ, 76mmØ, 102mmØ, 72x72mm, 102x76mm or 102x102mm TBC

Type / grade: Marine grade extruded aluminium to BS EN 755-2:2008

Finish:  Polyester powder coated to BS EN 12206-1:2004

Colour: TBC
	Functional life expectancy: 50 years

Jointing: Internal Snap fit joint union with integral silicone spacers and sealed with Alutec sealant
Fixing: BS EN 1462:2004 Class H, concealed fascia brackets at 750mm centres. Fixed with Alutec austenitic stainless steel 32mm x No. 10 round head austenitic stainless steel screws